DAB Active Driver Plus M/M 1.5 CAN Reverse-Engineering

Consolidated Test History, Evidence Chain, and Next-Step Rationale

Status: Working technical report based on the project documents, manuals, notes, metadata, passive captures, injection logs, token tests, and family-probe result files available on 28 September 2026.

Executive Summary

The investigation began with the documented fact that the DAB Active Driver Plus supports a CAN-connected multi-inverter system of up to eight units, with automatic or manual addresses, automatic detection of the number of connected inverters, configuration synchronization, and leader/start-order functions. Because only one inverter was available, the original strategy was to use repeated passive captures and controlled configuration changes to identify discovery and addressing traffic.

The passive captures established a highly regular proprietary CAN pattern: a 29-bit extended, 1 Mbit/s bus carrying a long 0x0001TTTT/DLC7 frame followed approximately 0.1–0.2 ms later by a short 0x0011TTTT/DLC5 frame with the same 16-bit token. The pair repeats approximately every 5 ms. The token is present in the lower 16 bits of the CAN identifier and is repeated little-endian in the payload. This disproved several early interpretations, including a checksum interpretation of the final bytes and a simple startup-state interpretation of marker 0x00.

Changing AD between automatic/0, 1, 2, and 3 did not measurably change the normal recurring traffic. This rejected the simplest hypothesis that the passive identifier prefixes directly reveal the locally configured AD address. Later listen-only and pump-run captures showed that the same beacon-like traffic continued without an active PCAN acknowledgment source and did not visibly encode pump operation. This moved the investigation away from passive process-data decoding and toward controlled peer/discovery stimulation.

The first broad injection campaign used fake-node pairs, alternate markers, fixed or random tokens, and long/short-frame bursts. It produced no display message, new fault, or stable protocol-state change. The decisive breakthrough came when a syntactically valid short frame was transmitted in family 0x0012: the device produced a repeatable burst on 0x103C0401 and 0x103C0481. Token variation then showed that the response was not specific to token C82D. Family scanning showed that, in a responsive AD=1/device state, families 0x0012 through 0x0018 were accepted, while 0x0002, 0x0009, 0x0010, and 0x0019 were not. Exactly seven accepted families correspond naturally to the seven possible peers of an eight-address system.

However, valid AD=2 runs with normal RX traffic produced no 0x103C response for any tested family. This means family syntax alone is not sufficient. Acceptance is state-dependent and may depend on AD configuration, discovery state, startup state, menu context, a previous association, or an internal timeout/lockout. The correct next step is therefore a tightly controlled state matrix, not a wider uncontrolled family sweep.

1. Documentary Basis

1.1 What the official manuals established

The product has CAN connectivity and a dedicated three-wire multi-inverter connection.

For M/M units, the multi-inverter connector is J1.

A multi-inverter group may contain up to eight pump/inverter elements.

Parameter AD is the communication address: automatic by default or manually assigned from 1 to 8.

All units in a group must use the same addressing method, and duplicate manual addresses are not permitted.

At power-up, automatic address assignment and leader selection occur in a multi-inverter system.

N is the automatically detected number of connected inverters.

The DConnect Box officially supports the Active Driver Plus over CAN, using a special cable with a 120-ohm termination at the DConnect end.

The installed Active Driver Plus software version recorded in the test metadata is 2.13, which meets the DConnect requirement for software version 2.x or later.

Why this led to passive startup testing: if automatic addressing, peer counting, and leader selection occur at startup, repeated power-up captures should expose deterministic discovery or address-allocation traffic.

2. Chronological Test Sequence

Stage A — December 2025 exploratory captures

Files: Test_001_AD-1.CSV through Test_009.CSV.

Reason for the tests: obtain initial CAN traffic, compare AD configurations, identify recurring frame structures, and try known-valid tokens in active transmissions.

Main observations:

Two frame forms repeatedly appeared with the same token: a seven-byte long form and a five-byte short form.

The token appeared in the CAN identifier and in the payload as reversed bytes.

Token C82D occurred naturally in both AD=1 and AD=2 captures.

In Test_009.CSV, C82D was deliberately reused in transmitted frames while the upper identifier family was varied.

Original interpretation: the long frame carried full status/data and the short frame might be a compact confirmation; marker 0x00 was thought to mean startup and 0x03 normal operation.

Why it led to the next step: the apparent frame pairing and the documented AD function justified a larger, controlled passive comparison across AD settings. C82D became a useful fixed probe token because it was demonstrably protocol-valid, not because it was assumed to be a special command.

Stage B — September 2026 passive startup series, Traces 001–014

Files: can_Trace_001_20260925_1435_AD-1.csv through can_Trace_014_20260928_1630_AD-0.csv, with the accompanying trace notes.

Conditions: 1 Mbit/s, 29-bit extended CAN, pump generally in standby at approximately 3.2 bar and 0.0 Hz. The files cover AD=0/automatic, AD=1, AD=2, and AD=3.

Measured result:

Every capture contained equal numbers of DLC7 and DLC5 frames.

The two identifier families were consistently the long-frame family 0x0001 and short-frame family 0x0011.

The recurring cycle was approximately 5 ms, or about 200 frame pairs per second.

The long and short messages shared the same 16-bit token.

The token was echoed little-endian in the payload.

Each of Traces 001–015 contained one marker-0x00 long frame; marker 0x03 dominated; marker 0x04 occurred infrequently. Traces 016–017 did not contain marker 0x00, demonstrating that it was not a universal startup phase marker.

AD=0/automatic, 1, 2, and 3 produced the same passive family structure and statistically similar traffic.

Hypotheses rejected or corrected:

Checksum hypothesis: rejected. The final two payload bytes are the identifier token in little-endian form.

Short-frame status-byte hypothesis: rejected. The relevant bytes span all 256 values because they are token bytes.

Marker 0x00 = startup state: rejected as a general rule.

Simple AD-to-prefix mapping: rejected. Setting AD=2 or AD=3 did not change the passive prefixes to 0x0002/0x0012 or 0x0003/0x0013.

CANopen node guarding: rejected because the observed traffic uses 29-bit extended identifiers rather than the expected 11-bit CANopen identifiers.

J1939 Address Claim: rejected because the identifier layout and random low-16 field do not form a credible J1939 priority/PF/PS/source-address structure.

Revised interpretation: the traffic is a proprietary periodic beacon or peer-management structure using a rapidly changing 16-bit correlation token. The upper identifier field is a family/class/slot field, but it cannot yet be called a source address.

Why it led to the next step: passive AD changes were exhausted. The remaining uncertainties were whether the PCAN adapter was the only ACK source, whether the traffic carried process data, and whether a peer had to be actively simulated.

Stage C — Controlled peer-injection campaign on 25 September 2026

Files: pcan_test_results chatbot - timeline_all.csv, phase-specific timeline files, tx_attempts.csv, rx_frames.csv, metadata, manual notes, and exported result CSVs.

Reason for the tests: test the safest available discovery/election hypotheses while monitoring the display and error history.

Execution: 1,608 successful transmissions and no failed writes were recorded. The campaign included:

Single fake-node pair with token 0x1234 and marker 0x00.

Single fake-node pair with marker 0x03.

One-second bursts of paired frames using fixed or changing tokens.

DLC7-only and DLC5-only periodic transmissions.

Paired DLC7/DLC5 bursts with the measured 114 µs separation.

Observed result:

No display message.

No new fault in FF history.

The communication icon remained “no communication.”

N remained 1 except for one manually noted transient N=0 observation during Test 23; this was not reproduced and was not accepted as a protocol finding.

The pump was deactivated during the injection tests.

The recurring background traffic resumed or continued without a durable state transition.

Interpretation: recreating the native 0x0001/0x0011 pair, changing its marker, or sending it rapidly was insufficient to create a recognized peer. The natural pair is therefore unlikely to be a complete standalone “node announcement” message.

Why it led to the next step: the negative result suggested that either the wrong family was being stimulated, the short frame rather than the long/short pair was the meaningful peer input, or an additional state condition was required. This motivated controlled token and family scans.

Stage D — Listen-only and real pump-operation captures, Traces 015–017

Files: can_Trace_015_20260928_2020_AD-0.csv, can_Trace_016_20260928_2027_AD-0.csv, can_Trace_017_20260928_2034_AD-0.csv, and Trace 15..17.txt.

Reason for the tests: determine whether the PCAN adapter had been the only ACK source and whether the captured channel contains pump process data.

Trace 15: capture started in listen-only before the pump unit was powered.

Trace 16: the pump had been powered for approximately five minutes before the listen-only CAN connection was made.

Trace 17: listen-only capture while a tap was opened and the pump started running.

Observed result:

The same paired 0x0001/0x0011 traffic remained present in listen-only mode.

The display continued to show N=1, AD=0, and “no communication.”

Trace 17, including an actual pump start, still contained only the same two dominant families and the same frame grammar.

No clear pressure, speed, run-state, or setpoint field emerged from the captured channel.

Interpretation: the periodic traffic is not obviously normal pump process telemetry. The continued traffic in listen-only means that the earlier normal-mode captures were not solely created by PCAN acknowledgments. A second internal CAN participant or a transmitter-specific error-handling behavior remains possible.

Why it led to the next step: because a real hydraulic state change did not reveal process data, the highest-value path became protocol discovery/peer stimulation rather than further idle or run captures.

Stage E — Token-independence tests on family 0x0012

Files: dab_token_test_v31_*_20260928-1325.csv and dab_token_test_v31_*_20260928-1329.csv, plus the single-probe ad0_* files.

Reason for the tests: determine whether the observed reaction was caused by the fixed token C82D or by the selected identifier family.

Method: family 0x0012 was tested with tokens 0001, 8000, FFFF, and C82D. The payload always contained the token in little-endian form.

Observed result: both repetitions produced the same response signature for all four token values:

0x103C0401, DLC4;

0x103C0481, DLC8;

normally 32–34 response frames in a burst of about 80 ms.

Only the 0001 probe naturally caused an exact token match to appear in one response payload; the other successful probes did not echo their token. Therefore, token echo is not required for acceptance.

Timing caveat: the v31 result files report a first latency of approximately 3.445 seconds. This is inconsistent with the raw family-probe captures, which show the 103C burst immediately after accepted probes. The 3.445-second value should be treated as a logger/reference-window timing defect, not device response latency.

Conclusion: C82D is a known-valid test constant, not a magic command. The family/class field is the discriminating variable.

Why it led to the next step: scan candidate family values while keeping token and payload fixed.

Stage F — Family scans with C82D

Files: all dab_family_probe_raw_*, dab_family_probe_results_*, and dab_family_probe_metadata_* files.

Probe grammar:

CAN ID: (family << 16) | 0xC82D

DLC:    5

DATA:   01 00 00 2D C8

F1. Initial responsive AD=1 runs

The 13:47 and 13:50 runs tested families 0x0010 and 0x0012–0x0015, three times each:

0x0010: 0/6 responses across the two runs.

0x0012, 0x0013, 0x0014, 0x0015: each responded on every probe.

Accepted probes generated 32–34 frames on 0x103C0401 and 0x103C0481, lasting approximately 78–94 ms.

The display reaction occurred together with the 103C burst.

Reason for expansion: determine the boundaries of the accepted family range and whether it aligns with the documented addresses 1–8.

F2. Invalid or non-interpretable runs

Several intermediate runs recorded successful TX attempts but zero RX frames. These include the 14:28, 14:32, 14:36, 14:39, 14:41, and 14:49 campaigns. Their zero-response outcome cannot be used as evidence that the device rejected those families, because the receive side was not operating. The 14:26 run captured only its initial baseline and no RX during the probe phase, so it is also unsuitable as a normal acceptance test.

Why this matters: these files are useful as script/adapter diagnostics, but they must not be mixed with valid negative application-layer results.

F3. Valid expanded responsive runs

The 14:55 and 14:57 valid runs had normal RX traffic and tested eleven families three times each. The repeated result was:

FamilyResultInterpretation

0x00020/6Rejected in responsive state

0x00090/6Rejected in responsive state

0x00100/6Rejected in responsive state

0x0012–0x00186/6 for each familyAccepted range

0x00190/6Rejected in responsive state

The device therefore accepted exactly seven consecutive families. The display reportedly changed exactly 21 times in the three-repeat run, matching seven accepted families multiplied by three repetitions.

Strongest current mapping hypothesis:

family = 0x0010 + logical address

0x0011 → address 1

0x0012 → address 2

...

0x0018 → address 8

With the local unit occupying or representing slot/address 1, families 0x0012–0x0018 would represent the seven possible peer slots. This is a strong but still unproven destination/slot interpretation.

F4. Valid AD=2 negative runs

The 15:03 and 15:06 AD=2 runs both had normal baseline and probe-window RX traffic, all 33 TX attempts succeeded, but no tested family generated a 103C response. This is a valid negative result, unlike the earlier TX-only runs.

Interpretation: the acceptance rule is not merely “family 0x0012–0x0018 is always valid.” It is better expressed as:

syntactically valid peer-family frame

+ correct device/configuration state

→ display reaction and 103C burst

Why it led to the next step: the decisive unknown is now the state variable controlling acceptance—possibly AD mode, address ownership, discovery activation, startup history, menu context, or a lockout after previous probes.

3. Current Protocol Model

3.1 Identifier layout

29-bit CAN identifier = (upper family/class/slot field << 16) | 16-bit token

The project currently calls the upper field “family,” but its final meaning is not proven.

3.2 Normal recurring pair

0x0001TTTT  DLC7  01 00 00 0F MM LL HH

0x0011TTTT  DLC5  01 00 00       LL HH

TTTT is the token.

LL HH is the token in little-endian payload order.

MM is a marker/state byte, usually 0x03, occasionally 0x04, and sometimes 0x00 near the start of a capture.

3.3 Accepted short peer/discovery probe

0x001nTTTT  DLC5  01 00 00 LL HH

Under the currently identified responsive state, n=2…8 produces the application response.

3.4 Response burst

0x103C0401  DLC4  00 00 00 01

0x103C0481  DLC8  00 00 00 00 00 10 XX 14

Normally 15 or 17 frames on 0x103C0401 and 17 on 0x103C0481.

Total burst: 32 or 34 frames.

Duration: approximately 78–94 ms.

XX changes slowly and is not a direct echo of the injected family.

4. Evidence-Based Conclusions

Demonstrated with high confidence

The physical/application capture is Classical CAN at 1 Mbit/s with 29-bit extended identifiers.

The recurring traffic is proprietary rather than a demonstrated CANopen or J1939 profile.

The long and short recurring frames form an intentional token-matched pair.

The low 16 identifier bits are a token repeated little-endian in the payload.

Passive AD changes do not alter the normal 0x0001/0x0011 frame families.

A valid short frame alone can trigger an application-layer reaction.

The reaction is associated with a 103C0401/103C0481 burst.

C82D is not uniquely required; other well-formed tokens also work.

In a responsive state, families 0x0012–0x0018 are accepted and adjacent control families are not.

Acceptance is state-dependent: valid AD=2 runs produced no 103C response.

Strong but not yet proven

Families 0x0011–0x0018 represent eight logical address, peer, or destination slots.

The local unit in the responsive AD=1 condition occupies slot/address 1, leaving 0x0012–0x0018 as possible peer slots.

The 103C burst represents discovery, temporary peer presence, UI/status refresh, or communication-state handling rather than a family-specific acknowledgment.

Not established

Whether the family field is a source address, destination address, slot, or message class.

The exact meaning of marker 0x04.

The exact meaning of 0x103C, 0x0401, 0x0481, and byte XX.

Whether a complete peer emulation requires periodic long and short messages after the initial accepted short frame.

Whether process data is carried on this connector in a one-unit, no-communication state.

Whether the same CAN application layer is used by DConnect and by peer-to-peer multi-inverter operation.

5. Why the Investigation Has Reached the Current Follow-Up Step

Manual evidence predicted addressing, discovery, peer counting, and leader selection.

Repeated passive captures found a proprietary beacon but no AD-dependent passive address field.

Statistical analysis showed the token was dynamic and the payload tail was an identifier echo, eliminating several incorrect byte-level interpretations.

General fake-node injection on the natural 0x0001/0x0011 pair did not create a peer, so the native pair was not a sufficient discovery announcement.

Listen-only and pump-run tests showed that the captured beacon continued without an active PCAN ACK source and did not visibly carry normal process data.

Token tests showed the decisive variable was the family/class field, not C82D.

Family scans found a precise accepted range corresponding to seven possible peers.

AD=2 repetitions removed the idea of an unconditional family rule and exposed a state-dependent acceptance gate.

The next experiment must therefore isolate that gate. More passive idle captures or wider family sweeps would add volume without resolving the controlling state.

6. Recommended Next Test Sequence

Priority 1 — Controlled AD/state matrix

Use one known accepted probe, preferably family 0x0015 with token C82D, and change only one condition at a time:

StepConditionPurpose

1Power cycle, AD=1, no previous probesReconfirm the responsive reference state

2Repeat three times after full power cycleEstablish reproducibility

3Change only AD to 2, then full power cycleTest whether address ownership gates acceptance

4Return to AD=1, full power cycleTest reversibility and exclude time/lockout effects

5Repeat with AD=0/automatic and AD=3Build the complete acceptance matrix

For every run, record AD, N, communication icon, menu page, pump enabled/deactivated state, time since power-up, pressure, frequency, error history, listen-only/normal mode, and whether the device has already received probes.

Priority 2 — Determine the peer-slot rule

If the AD/state matrix is reproducible, test all families 0x0011–0x0018 for each local AD value. The key prediction is:

If the family is a destination/peer slot, the locally occupied slot may be excluded while the other seven are accepted.

If the accepted set remains fixed at 0x0012–0x0018 regardless of AD, the field is more likely a message class or fixed role range.

Priority 3 — Hold the reaction long enough to expose higher-layer traffic

After an accepted short probe, transmit a conservative periodic presence pattern for one candidate peer and observe whether:

N changes from 1 to 2;

the communication icon changes;

new fixed identifier families appear;

parameter synchronization or leader-selection traffic starts;

the 103C burst becomes sustained rather than temporary.

This should be attempted only after the one-shot acceptance rule is reproducible and with the pump hydraulically safe.

Priority 4 — DConnect Box capture

The DConnect Box is the best documented external peer. Connect it with the official terminated cable and passively capture:

DConnect boot;

device search;

successful Active Driver Plus recognition;

one parameter read;

one harmless parameter change and restoration.

This would provide known request/response semantics and may reveal identification, firmware, parameter, and discovery messages that cannot be produced with one standalone inverter.

Priority 5 — Electrical verification

Confirm the M/M multi-inverter connection is J1.

Measure termination with power removed.

Verify CAN-H, CAN-L, and the centre shield/GND connection.

Document whether the observed segment has one or two terminators.

7. Data-Quality and Documentation Corrections

Several metadata files and CSV files are duplicates with and without an AD suffix. They should be treated as duplicate exports, not independent tests.

The intermediate family-probe runs with total_rx = 0 are TX/logging diagnostics and cannot support application-level rejection conclusions.

The 14:26 run captured baseline RX only and no probe-window RX; its zero-response result is not directly comparable to valid runs.

The v31 token-test latency of approximately 3.445 seconds is a timing-reference artifact; the later raw family-probe data show an immediate response burst.

Trace 013 and Trace 014 contain 20260928 in their filenames while the directory notes list 25 September 2026. The acquisition date should be corrected or explicitly annotated.

The early note that “byte 4 is startup status and the last bytes are checksum” is superseded by the later token-echo analysis.

The original statement that the bitrate was unknown is superseded: the successful captures and script metadata establish 1 Mbit/s.

8. Source Set Reviewed

The consolidation used the available project documentation and data groups, including:

DAB active Driver.docx

DAB Active Driver Plus CAN Reverse Engineering Test Plan.docx

Safe discovery probes.docx

Token C82D.docx

Reverse-engineering CAN-protocol.txt and Reverse-engineering CAN-protocol-1.txt

Tests DAB AD protocol.txt

Test aan de hand van het inverter adres.docx

Stappenplan vervolgonderzoek.txt and Stappenplan vervolgonderzoek new.txt

Trace_001 to 006.txt, Trace_007 to 014.txt, and Trace 15..17.txt

OBV Active Driver Plus.pdf

OBV D.connect-3.pdf

December 2025 Test_001–Test_009 captures

September 2026 passive CAN traces 001–017

The complete peer-test timeline, phase, RX, TX, result, metadata, and manual-note files

The AD0 probe, token-test v31, and family-probe raw/result/metadata files

Final working conclusion: the project has progressed from an undifferentiated random-looking beacon to a preliminary application-layer grammar and a repeatable peer-family acceptance phenomenon. The immediate research objective is no longer “find any response.” It is now “identify the device state and address rule that enable or suppress the accepted 0x0012–0x0018 peer/discovery response.”