DAB Active Driver Plus CAN Application Layer

Preliminary Protocol Specification — Version 0.1

Status: reverse-engineered working draft, 28 September 2026

1. Purpose and Scope

This document summarizes the currently known layout and organization of the proprietary CAN application layer used by the DAB Active Driver Plus. It separates demonstrated observations from working hypotheses. Several field meanings remain unknown and should not yet be treated as final protocol definitions.

2. Protocol Profile

PropertyCurrent conclusionConfidence

Physical layerCAN, successfully tested at 1 Mbit/sHigh

CAN frame format29-bit extended framesDemonstrated

Application protocolProprietary DAB protocolHigh

CANopen/J1939Not demonstrated; the observed layout does not directly match either protocolHigh

CAN generationClassical CAN, with payload lengths of up to 8 bytesDemonstrated

Dominant cycle timeApproximately 5 ms, or 200 Hz, per recurring frame familyDemonstrated

Documented addressesAutomatic addressing or manually assigned addresses 1–8Documented

CRC/checksumNo separate CRC or checksum field has yet been identifiedUnknown

The DAB manual confirms that parameter AD supports automatic addressing or manually assigned addresses from 1 through 8. Source: OBV Active Driver Plus.pdf.

3. 29-bit CAN Identifier Layout

The most useful current interpretation is:

Bit 28                         Bit 16  Bit 15                       Bit 0

┌───────────────────────────────────┬──────────────────────────────────┐

│ Family / class / address field    │ 16-bit token                     │

│ 13 bits, displayed as 4 hex digits│                                  │

└───────────────────────────────────┴──────────────────────────────────┘

CAN ID = (family << 16) | token

Example:

CAN ID: 0x0015C82D

Family: 0x0015

Token:  0xC82D

The lower 16 bits of the CAN identifier therefore contain the token directly.

3.1 Terminology qualification

The capture software currently calls the upper field the family. Its final meaning has not been established. It may ultimately represent one or more of the following:

message class;

source or destination address;

node slot;

communication channel;

function code.

The fact that probes in families 0x0012–0x0018 are accepted does not by itself prove that this field is exclusively a source address.

4. Normal Recurring Frame Pairs

The controller continuously produces two closely coupled frames that share the same token.

4.1 Long frame

CAN ID: 0x0001TTTT

DLC:    7

DATA:   01 00 00 0F MM LL HH

Where:

TTTT is the 16-bit token in the CAN identifier;

LL is the least significant token byte;

HH is the most significant token byte;

MM is a marker or state byte.

Example:

CAN ID: 0x0001D341

DATA:   01 00 00 0F 03 41 D3

Token in CAN ID:  0xD341

Token in payload: 41 D3, little-endian

Marker:           0x03

4.2 Short frame

CAN ID: 0x0011TTTT

DLC:    5

DATA:   01 00 00 LL HH

For the same token:

CAN ID: 0x0011D341

DATA:   01 00 00 41 D3

4.3 Relationship between the frames

Long frame:  0x0001TTTT, DLC 7

Short frame: 0x0011TTTT, DLC 5

Both frames have:

the same 16-bit token;

the same first three payload bytes, 01 00 00;

the token encoded little-endian in the payload;

a very short interval between them;

a recurrence interval of approximately 5 ms.

0x0001TTTT  01 00 00 0F MM LL HH

│

│ approximately 0.1–0.2 ms

▼

0x0011TTTT  01 00 00 LL HH

The family offset between the long and short frames is:

0x0011 − 0x0001 = 0x0010

This strongly indicates an intentional protocol pair rather than two coincidentally similar messages.

5. Fixed Payload Fields

5.1 Initial bytes

01 00 00

These bytes remain constant in nearly all observed presence/discovery-type frames. Possible functions include a protocol version, message type, header, status word, or reserved fields. Their exact meaning remains unknown.

5.2 Byte 0x0F

The long frame nearly always contains 0F after the first three bytes:

01 00 00 0F MM LL HH

└─ probable operation code, message group, or subfunction

5.3 Marker byte MM

MarkerObservationPreliminary interpretation

00Frequently observed early during startupInitialization or start marker

03Dominant during normal operationNormal operational state

04Sporadic and transientEvent or state transition

05Observed incidentallyInsufficient evidence

The normal recurring frame is predominantly:

01 00 00 0F 03 LL HH

6. The 16-bit Token

The token is redundantly encoded in two places:

CAN ID lower 16 bits: TTTT

Payload:              LL HH

Example:

Token:   C82D

Payload: 2D C8

Current observations indicate that the token:

changes rapidly;

is nearly always new in normal captures;

is shared by the associated long and short frames;

is therefore likely a transaction, discovery, challenge, or correlation token;

is unlikely to be a permanent serial number or static node address.

The redundancy may support pair correlation, simple integrity checking, arbitration-ID distribution, collision avoidance during discovery, or request/response matching. No separate traditional CRC has yet been identified.

7. Active Short-Frame Probe

The active probe uses the same structure as the normal short frame:

CAN ID: 0xFFFFTTTT

DLC:    5

DATA:   01 00 00 LL HH

For token 0xC82D:

DATA: 01 00 00 2D C8

Example:

CAN ID: 0x0015C82D

DATA:   01 00 00 2D C8

The controller can therefore respond to a syntactically correct short frame even when the corresponding long 0x0001 frame was not injected first. The short frame alone is sufficient to trigger an application-layer reaction under the correct operating conditions.

8. Family or Slot Range

In dab_family_probe_results_20260928-145745.csv, the observed result was:

FamilyResult across three probes

0002No 103C response

0009No 103C response

0010No 103C response

00123/3 responses

00133/3 responses

00143/3 responses

00153/3 responses

00163/3 responses

00173/3 responses

00183/3 responses

0019No 103C response

This produced exactly:

7 families × 3 repetitions = 21 response events

The display was also observed to change 21 times.

8.1 Address mapping hypothesis

Combined with the documented AD addresses 1–8, the strongest current hypothesis is:

Logical address 1 → family 0x0011

Logical address 2 → family 0x0012

Logical address 3 → family 0x0013

Logical address 4 → family 0x0014

Logical address 5 → family 0x0015

Logical address 6 → family 0x0016

Logical address 7 → family 0x0017

Logical address 8 → family 0x0018

Possible formula:

family = 0x0010 + logical address

8.2 Important limitation

Passive captures made with AD settings 0, 1, 2, and 3 still show the same normal frame prefixes:

0x0001

0x0011

The prefixes do not simply change to 0x0002/0x0012 or 0x0003/0x0013 when the local AD setting changes. It is therefore not yet justified to say that 0x0011 is unequivocally the source address of the installed controller.

A more accurate conclusion is:

Families 0x0011–0x0018 appear to be valid peer, slot, or destination families corresponding to the eight possible AD addresses. It has not yet been established which bits encode source address, destination address, or message class.

9. 0x103C Response Structure

An accepted probe causes a short burst on two fixed CAN identifiers:

0x103C0401

0x103C0481

The identifier decomposition appears to be:

CAN ID: 0x103C0401

Family: 0x103C

Low 16: 0x0401

CAN ID: 0x103C0481

Family: 0x103C

Low 16: 0x0481

The lower field may itself be divided into 04 01 and 04 81. The difference of 0x80 between 01 and 81 may indicate direction, request versus response, two channels, status versus data, or acknowledged versus broadcast traffic. This is not yet proven.

9.1 Response 0x103C0401

CAN ID: 0x103C0401

DLC:    4

DATA:   00 00 00 01

This payload remained constant during the successful run. Possible functions include presence confirmation, acknowledgment, count/status value 1, or a general communication state.

9.2 Response 0x103C0481

CAN ID: 0x103C0481

DLC:    8

DATA:   00 00 00 00 00 10 XX 14

Observed payloads included:

00 00 00 00 00 10 61 14

00 00 00 00 00 10 62 14

00 00 00 00 00 10 63 14

Preliminary byte layout:

Byte:    0  1  2  3  4  5  6  7

DATA:   00 00 00 00 00 10 XX 14

│

└─ slowly changing value

The XX byte does not directly reflect the injected family. It is more likely to be a counter, internal state, display-related value, discovery state, or event count. Byte 7 remained 0x14 during this run.

10. Response Burst and Timing

Each accepted probe normally produces:

0x103C0401: 15 or 17 frames

0x103C0481: 17 frames

Total:      32 or 34 frames

The burst lasts approximately:

78–94 ms

This is consistent with a period of approximately 5 ms:

17 frames × approximately 5 ms ≈ 80 ms

The application response is therefore not a single acknowledgment frame. It is a short periodic burst:

Accepted probe

│

▼

Display changes

│

├── 0x103C0401 / DLC 4

└── 0x103C0481 / DLC 8

repeated for approximately 80 ms

Because the same 103C response appears for all accepted families and the injected family is not echoed in the response payload, this is more likely a general UI/status update, discovery event, or temporary communication state than a family-specific acknowledgment.

11. Probable State Dependence

The same family probes did not produce the same reaction under every configuration.

During a run labeled AD=1:

0x0010 → 0/3 responses

0x0012 → 3/3 responses

0x0013 → 3/3 responses

0x0014 → 3/3 responses

0x0015 → 3/3 responses

During a later run labeled AD=2:

0x0002, 0x0009, 0x0010, 0x0012–0x0019 → no 103C responses

Normal receive traffic was present during the AD=2 run, but the 103C response was absent. Acceptance therefore probably depends on more than the CAN identifier alone. Relevant factors may include:

AD configuration;

automatic versus manual addressing;

active or inactive communication state;

startup phase;

current fault state;

display or menu context;

a previously discovered peer;

an internal timeout or lockout.

The current rule is therefore:

Syntactically valid family

+ correct device state

→ display change and 103C burst

It is not yet valid to conclude that families 0x0012–0x0018 always produce a response.

12. Current Logical Protocol Model

Normal controller cycle

│

├─ 0x0001TTTT / DLC 7

│      header + opcode + state marker + token

│

└─ 0x0011TTTT / DLC 5

header + token

Peer/discovery injection

│

└─ 0x001nTTTT / DLC 5

n probably relates to slot/address 1–8

token in CAN ID and little-endian payload

Accepted peer/discovery event

│

├─ display changes

├─ 0x103C0401 / DLC 4 / 00 00 00 01

└─ 0x103C0481 / DLC 8 / 00 00 00 00 00 10 XX 14

13. Remaining Unknowns

Whether 0x0011–0x0018 are source addresses, destination addresses, or slot identifiers.

Why runs labeled AD=2 did not produce a 103C response.

The exact meaning of family 0x103C.

The meanings of lower identifier fields 0x0401 and 0x0481.

The meaning of byte 6, observed as 61, 62, or 63, in the DLC 8 response.

The event represented by marker 0x04.

Whether the token has functional meaning or only provides correlation.

Whether a complete long-plus-short frame pair has a greater effect than the short frame alone.

The structure of parameter, status, and runtime messages.

Whether sequence counters, checksums, or authentication fields appear in other protocol states.

14. Condensed Preliminary Grammar

29-bit CAN ID:

[13-bit family/class/slot][16-bit token]

Normal long frame:

0x0001TTTT  01 00 00 0F MM LL HH

Normal short or peer frame:

0x001nTTTT  01 00 00 LL HH

Token representation:

CAN ID low16 = TTTT

Payload       = little-endian LL HH

Accepted discovery or peer probe response:

0x103C0401  00 00 00 01

0x103C0481  00 00 00 00 00 10 XX 14

The evidence is now sufficient to define an initial protocol grammar. It is not yet sufficient to identify the family field definitively as a node address.

15. Sources

OBV Active Driver Plus.pdf

Reverse-engineering CAN-protocol.txt

dab_family_probe_results_20260928-145745.csv

dab_family_probe_raw_20260928-145745.csv

dab_family_probe_results_20260928-134706_AD=1.csv

dab_family_probe_results_20260928-150320_AD=2.csv

can_Trace_001_20260925_1435_AD-1.csv

can_Trace_008_20260925_1616_AD-0.csv

can_Trace_010_20260925_1621_AD-2.csv

can_Trace_012_20260925_1625_AD-3.csv