Integrated Reconstruction — DAB Active Driver Plus CAN

Status: First integrated review of the complete investigation file, 30 September 2026Objective: Determine what has been demonstrated, disproved, exhausted, and what remains genuinely promising for recovering CAN telemetry without purchasing additional equipment.

1. Executive conclusion

The investigation has now reconstructed much of the spontaneous multi-inverter discovery and network-management layer. That work produced substantial, reproducible results, but it is no longer the principal bottleneck. Continuous telemetry—pressure, frequency, current, power, operating state, faults, and parameters—has not been demonstrated in the spontaneous 0001/0011 traffic.

The DConnect Box documentation does, however, establish that a DConnect Box uses the same three-wire Active Driver Plus connection to:

search for and identify a connected Active Driver Plus;

display its principal operating parameters;

change parameters remotely;

provide historical graphs;

and update older Active Driver Plus firmware.

This leads to the central conclusion:

The required telemetry is probably not an unrecognised passive broadcast hidden in the existing beacon frames. It is more likely exposed only after a product-specific DConnect request, session, or polling layer has been established.

The best route that does not require purchasing hardware is therefore not another broad CAN sweep. It is software archaeology focused on DConnect: GPL corresponding source, update packages, application resources, cloud/API schemas, and firmware manifests. A new CAN experiment is justified only after this produces a concrete identifier, request payload, parameter index, or protocol hypothesis.

2. Source hierarchy

2.1 Normative documentation

OBV Active Driver Plus.pdf

OBV D.connect-3.pdf

Official DConnect Box manual

DAB MCE/P product information

Publicly available copy of the DAB Modbus manual

2.2 Experimental evidence

The principal evidence comes from:

passive traces 001–017;

peer, short, pair, and replay tests;

the family probes performed on 28 September;

the hierarchy tests performed on 29 September;

and the AD-map, smart-banks, brute-force, boundary, alias, token-sweep, node-minimal, P4, and gateway tests performed on 30 September.

2.3 External software

ankohanse/hass-dab-pumps

ankohanse/pydabpumps

Home Assistant discussion concerning the DAB integration

These projects are relevant to DAB cloud semantics and exposed entities. At present, they do not constitute evidence for the local Active Driver Plus CAN format.

3. Evidence ledger

3.1 Demonstrated physical and data-link layer

ClaimStatusBasis

The physical layer is CANDemonstratedThe manual identifies CAN connectivity and the same three-wire connector for other Active Driver Plus units and DConnect.

Bitrate is 1 Mbit/sDemonstrated in the test setupAll recent captures and active tests operate reliably at 1,000,000 bit/s.

29-bit extended identifiersDemonstratedAll relevant traffic uses extended CAN identifiers.

Classical CAN, not CAN FDDemonstratedObserved DLC does not exceed eight bytes.

CANopen or J1939 as the direct application layerRejected as working modelsThe identifier and traffic structures do not fit CANopen; a J1939 interpretation does not produce a viable addressing model.

3.2 Spontaneous traffic from the single inverter

The dominant spontaneous traffic has the following structure:

0x0001TTTT  DLC 7  01 00 00 0F MM LL HH

0x0011TTTT  DLC 5  01 00 00       LL HH

TTTT equals HHLL; the payload contains the value in little-endian order as LL HH.

Long/short pairing is nearly complete, with the second frame typically following approximately 0.1–0.2 ms later.

The cycle interval is approximately 5 ms, or about 200 Hz.

MM=03 dominates; 00, 04, and occasionally 05 are exceptional values.

The lower 16-bit value changes almost uniformly and is not a permanent address or telemetry value.

Passive captures with AD set to AUTO, 1, 2, and 3 did not systematically alter this stream.

Conclusion: This traffic most likely supports presence announcement, discovery, collision avoidance, or network management. Its content is too limited to directly carry the documented set of FR, VP, C1, PO, VF, TE, BT, HO, EN, SN, fault, and configuration data.

3.3 Token C82D

C82D was reasonably selected as a known-valid reference token, but it does not have a unique command meaning.

It appeared naturally with both AD=1 and AD=2.

It was later held constant while the family field was varied.

The token sweep on family 0012 generated responses for all 32 tested values, with five repetitions per value: 160 responses from 160 probes.

The tested values included 0000, 0001, 1234, 7FFF, 8000, C82D, and FFFF.

Closed path: Brute-forcing tokens against this handler no longer has investigative value.

3.4 Valid short-family space

The complete brute-force run from 0000 through 1FFF found a 103C reaction only for:

0012–0018

0812–0818

Boundary and alias tests confirmed:

0011 and 0811: no response;

0012–0018 and 0812–0818: response;

0019, 001A, 0819, and 081A: no response;

+0x0800 acts as an alias or don't-care bit for this handler.

Closed path: Further broad family sweeps using the same short-frame grammar are not useful.

3.5 AD response mapping

The AD-map test produced the following response pairs for addresses 1–8, with three successful repetitions in each case:

ADStatus IDData ID

1103C0401103C0481

2103C0411103C0491

3103C0421103C04A1

4103C0431103C04B1

5103C0441103C04C1

6103C0451103C04D1

7103C0461103C04E1

8103C0471103C04F1

status_low16 = 0x0401 + ((AD - 1) << 4)

data_low16   = status_low16 + 0x80

The AD-matrix test also confirms that the response payload contains the local AD address. One AD=4 result decoded as address 3; this should be treated as a transition, timing, or operator anomaly rather than as invalidation of the otherwise consistent mapping.

3.6 Short, long, and pair behaviour

The node-minimal test has already established:

short only: the display showed “Wait...”;

long only: no visible effect;

pair: the communication icon flashed only at the start;

reverse pair: the communication icon also flashed only at the start.

The short-frame form is therefore the trigger for the known discovery/status handler. A long frame is not required for this reaction.

3.7 Hierarchy and N behaviour

The hierarchy tests show that simulated controllers genuinely influence the inverter's displayed unit count:

one simulated address generally produced N=1;

two simultaneous simulated addresses—3+5 and 2+3—produced N=2;

after stopping one simulated address, N returned to 1;

with two simulated addresses, VP=0.0 was observed;

one early address-3 run produced “Error”, while later repetitions were more stable.

Conclusion: The recovered grammar genuinely belongs to multi-inverter discovery and topology management. It is not merely an incidental display reaction. This does not yet demonstrate the DConnect telemetry-polling layer.

3.8 Gateway test

With the pump stopped and three repetitions per family:

0011: 0/3 responses;

0012: 3/3 responses, 100 response frames in total;

0811: 0/3 responses;

0812: 3/3 responses, 102 response frames in total;

0019: 0/3 responses;

no late 103C frames;

no communication icon remaining active;

no pump start.

Conclusion: The short probe creates a temporary discovery reaction, not a persistent DConnect session.

3.9 P4 and process state

Exact state labels and operator observations already exist. New manual state marking is unnecessary.

The first P4 capture contains only the normal 0001/0011 stream, the experiment's own 0012 polls, and temporary 103C bursts. In the second P4 run, responses occurred in 0/5 trials with the pump off, 0/5 at 2.0 bar, 1/5 at 2.2 bar, and 2/5 at 2.5 bar. This inconsistency is insufficient to support a telemetry claim and primarily indicates state or timing dependence in the handler.

Important: P4 has not demonstrated that pressure or frequency is encoded in the spontaneous beacon payload.

4. Documentation corrections

4.1 Older project notes are historical, not current

Reverse-engineering CAN-protocol.txt still states that the bitrate, CAN identifiers, and payload structure are unknown. It documents the starting point of the investigation and must not be used as the current status.

4.2 No separate pump-side commissioning wizard is documented

The Active Driver Plus manual describes:

a normal first-start wizard for LA, MS, SP, FN, UN/RC, and RT;

individual configuration of each inverter;

simultaneous power-up after individual configuration;

automatic addressing and leader selection when a multi-inverter system starts.

It does not describe a separate multi-inverter commissioning or association wizard on the Active Driver Plus. The earlier step 3A in the follow-up plan was therefore based on an unsupported extrapolation.

The DConnect application does have a configuration wizard that searches for connected devices. That is a gateway/application process, not a pump-side multi-inverter wizard.

5. What the DConnect documentation adds conclusively

5.1 The same physical Active Driver Plus connector is used

The documentation explicitly states that the three-wire connector is used for communication with:

other Active Driver Plus units;

and/or the DConnect Box.

For the M/M model, the Active Driver Plus manual identifies this as connector J1. The special DConnect cable includes a 120 Ω terminating resistor on the DConnect side.

5.2 Product-specific gateway ports

DConnect Box v1 provides:

CANBUS1-A/B for ADAC/MCE;

CANBUS1-C for Active Driver Plus;

CANBUS2-A/B for ADAC/MCE;

a separate Modbus port for Evoplus and fire-fighting products;

USB for EBox and expansion devices.

The parallel CANBUS1 ports may only be used with identical devices. Active Driver Plus and ADAC/MCE must not be connected simultaneously to the same parallel port group.

Consequence: DAB Modbus RTU is not automatically the transport used by Active Driver Plus. The Modbus documentation is primarily a semantic reference for names, data types, and scaling. Active Driver Plus uses the product-specific CAN path.

5.3 DConnect can both read and write

The application and documentation support:

viewing status parameters;

changing and transmitting configuration parameters;

historical graphs;

device discovery;

firmware recognition and updating.

The Active Driver Plus manual identifies the following locally available information:

FR — operating frequency;

VP — pressure;

C1 — phase current;

PO — power;

VF — flow present/absent;

TE/BT — temperatures;

FF — fault history;

HO — operating hours;

EN — energy;

SN — number of starts;

VE — hardware and software version.

This is the minimum expected telemetry model that the DConnect driver for Active Driver Plus must understand.

5.4 Firmware version is a protocol boundary

DConnect requires Active Driver Plus software version 2.x or later for normal recognition. Version 1.x can be updated through DConnect in “Loader LV” mode. The Active Driver Plus manual also describes firmware comparison and transfer between inverters.

At least two protocol layers therefore exist:

normal discovery, configuration, and runtime communication;

loader and firmware transfer.

The investigated inverter's VE version must therefore be explicitly recorded in the main investigation file.

6. FCC records, open-source notices, and ankohanse

6.1 FCC records

The DConnect manual lists 2ABCB-WLU6331 and OA3MRF24J40MA. These identify Wi-Fi and Microchip 2.4-GHz/Zigbee modules respectively. The FCC filings may provide internal photographs and module identification, but they do not directly identify the Active Driver Plus CAN daemon.

FCC filing OA3MRF24J40MA

6.2 GPL and other FOSS

DConnect Box v1 explicitly lists GPLv2, LGPLv2.1, OpenSSL, ZPL, Atmel, BSD, Apache, and MIT licences and points to:

http://dconnect.dabpumps.com/GPL

This does not guarantee that the proprietary CAN driver is available as source code. It may nevertheless reveal:

platform and kernel information;

CAN-interface configuration;

service and daemon names;

initialisation scripts;

firmware and update paths;

library versions and processor architecture;

debug strings or references to device profiles.

6.3 ankohanse

The public projects communicate with H2D, DConnect, or DAB Live backend services. Their direct value to this investigation is:

API endpoints;

authentication flows;

device and installation objects;

official entity names;

units and enumerated values;

potential model and firmware identifiers.

They do not demonstrate the local CAN framing, but they can provide the vocabulary needed to recognise strings and fields in DConnect software.

7. Closed, demonstrated, and genuinely open questions

7.1 Closed or exhausted paths

CANopen or J1939 as the direct protocol.

C82D as a magic value.

The lower 16 bits as a permanent address.

A complete long/short pair as a requirement for triggering the 103C response.

Further token sweeps on family 0012.

Further blind family sweeps using the same short-frame grammar.

A separate multi-inverter wizard on the Active Driver Plus as the missing prerequisite.

Repeating P4 merely to mark state transitions again.

7.2 Demonstrated, but not the final objective

Peer/discovery families 0012–0018.

Alias families 0812–0818.

103C address mapping for AD 1–8.

The short-only frame triggers the handler.

Simulated nodes influence N and the system display.

7.3 Genuinely open questions

The first DConnect request after device discovery.

Product identification and VE version over CAN.

Polling of FR, VP, C1, PO, VF, temperatures, operating hours, energy, and faults.

Parameter read and parameter write operations.

Whether DConnect activates a second family or class space after discovery.

Session maintenance, timeouts, and possible sequence fields.

The loader and firmware-transfer protocol.

The exact meaning of family bits outside the demonstrated short-frame handler.

8. Decision on the next step

The next step should not be another CAN probing script. The highest information value without purchasing equipment lies in obtaining and analysing existing software artefacts.

Work package A — Fully mirror and inventory the ankohanse repositories

Clone pydabpumps and hass-dab-pumps, including their complete history, tags, and releases.

Extract all endpoints, JSON keys, entity names, units, model codes, and fault codes.

Create a vocabulary mapping FR, VP, C1, PO, VF, TE, BT, FF, HO, EN, SN, and VE to DAB cloud terminology.

Check whether older commits contain DConnect-specific endpoints or product profiles that were later removed.

Work package B — Recover the GPL corresponding source

Try the published URL over both HTTP and HTTPS.

Search the Internet Archive and search-engine caches.

Search for the exact licence statement, product code 60183883, “DConnectBox”, “GPL”, “CANBUS1”, and likely archive filenames.

Submit a written request to DAB for the GPL/LGPL corresponding source for DConnect Box v1 and Box2.

Work package C — Application and update artefacts

Locate historical Android DConnect APK versions corresponding to Box v1.

Extract resources, strings, API routes, model tables, and update URLs.

Search for firmware manifests and download filenames for DConnect and Active Driver Plus.

Search all binaries and resources for ASCII and hexadecimal references including 103C, 0401, 0481, 0012, CANBUS, ADPLUS, ACTIVE_DRIVER, FR, VP, C1, PO, and VE.

Work package D — Only then return to targeted CAN validation

A new bus experiment should return to the agenda only when a software artefact yields at least one of the following:

a concrete arbitration identifier;

a request payload;

a product or profile identifier;

a polling interval;

a parameter index or register mapping;

a recognisable checksum or sequence function.

Only that hypothesis should then be tested, with one variable and a predefined success criterion.

9. Final direction

Stop blind CAN fuzzing. Begin DConnect software archaeology.

The existing CAN data is sufficient to validate newly discovered software information immediately. What is missing is not a larger dataset, but the request grammar used by the gateway that demonstrably exposes status, configuration, graphs, and firmware through the same physical connection.