Technical Working Document

# Reverse Engineering the DAB CAN-bus protocol

## DAB Active Driver Plus M/M 1.5

Status: Initial research framework

Purpose: Reconstruction of an undocumented CAN application protocol

Prepared for technical investigation and experimental validation

# Document Control

| Document title | Reverse Engineering the CAN Protocol — DAB Active Driver Plus M/M 1.5 |
| --- | --- |
| Document type | Technical research plan and working hypothesis |
| Protocol status | Undocumented; reconstruction not yet complete |
| Primary equipment | DAB Active Driver Plus M/M 1.5; PEAK PCAN-USB |
| Operating system | Windows 10 |
| Initial acquisition tool | PCAN-View |

# Executive Summary

The objective of this project is to investigate and reconstruct the undocumented CAN protocol used by the DAB Active Driver Plus M/M 1.5. Only one physical Active Driver Plus is currently available. The device combines the pump controller and variable frequency drive in one unit, while its communication interface is intended to support coordinated operation of multiple Active Drivers.

Official documentation confirms multi-inverter communication, addressing, automatic detection, configuration synchronization, and leader or priority functions. It also documents a three-wire communication connection and a 120 Ω termination resistor in the DConnect cable. These findings strongly support the use of a CAN physical layer, but they do not define the application protocol.

The current working hypothesis is that DAB uses a proprietary application protocol on top of CAN. The immediate objective is to obtain repeatable, completely passive startup captures in the original PCAN-View .TRC format before attempting semantic interpretation or active probing.

# Contents

In Microsoft Word, insert an automatic Table of Contents here using References → Table of Contents. The heading hierarchy in this document is suitable for automatic TOC generation.

# 1. Project Objective

The objective is to investigate the unknown CAN protocol used by a DAB Active Driver Plus M/M 1.5.

The situation is unusual because currently only one physical Active Driver Plus is available. The device combines the pump controller and variable frequency drive (VFD) in a single unit. The CAN interface is intended to enable multiple Active Drivers to work together in one system.

No complete public CAN protocol specification has been found. The protocol must therefore be reconstructed using official documentation and passive CAN captures.

# 2. Established Facts

## 2.1 Device

The device under investigation is the DAB Active Driver Plus M/M 1.5.

The Active Driver Plus is an inverter for pump control. DAB explicitly describes a communication interface for systems containing multiple Active Drivers, with one Active Driver Plus installed per pump.

DAB also provides official training covering:

- connection of a multi-inverter system;

- configuration of the Active Driver Plus;

- installation and commissioning.

## 2.2 Multi-Inverter Functionality

The Active Driver Plus is designed to work with other Active Drivers. Available DAB documentation indicates that the multi-inverter system includes provisions for:

- communication addresses;

- automatic detection of connected inverters;

- configuration synchronization;

- coordinated control;

- priority and leader functionality.

This confirms that the CAN traffic is not intended solely for basic logging or service access.

# 3. Physical Communication Layer

## 3.1 Physical Connection

The Active Driver Plus has a three-pin communication connection. For connection to the DConnect Box, DAB documents:

- two communication conductors;

- a GND/shield connection;

- a 120 Ω termination resistor between A and B at the DConnect Box end of the special cable.

These details provide strong evidence that the external interface uses a CAN physical layer.

Open item: The exact CAN bit rate has not yet been established.

## 3.2 DConnect Box

DAB offers an official DConnect Box that can communicate directly with the Active Driver Plus. The official documentation explicitly describes a “connection between Active Driver Plus and DConnect Box” using a special communication cable containing a 120 Ω termination resistor between A and B at the DConnect Box end.

A DConnect Box may potentially serve as a second communication participant without requiring a second Active Driver:

Active Driver Plus

│

│ CAN

│

DConnect Box

Communication between the devices could potentially be logged passively. This is a possible subject for further investigation.

# 4. Expected Protocol Functions

## 4.1 Automatic Detection and Addressing

The Active Driver Plus documentation describes an AD parameter as the communication address. Both automatic and manual addressing are possible, and manual addresses fall within the range of available multi-inverter nodes.

This suggests that a startup mechanism must exist through which units:

- announce their presence;

- detect other units;

- use or are assigned a communication address.

The exact CAN frames used for this process remain unknown.

## 4.2 Automatic Unit Count

The documentation uses the parameter N for the automatically detected number of inverters. It also defines parameters for the number of active inverters and the maximum number of inverters that may operate simultaneously.

CAN startup

│

▼

Unit detection

│

▼

number of units = N

│

▼

multi-inverter configuration

With one unit available, the standalone system is expected to identify itself as one unit. The exact mechanism used to determine N has yet to be established.

## 4.3 Leader and Priority Mechanism

DAB documentation explains that one inverter in a multi-inverter system can be designated as the settings leader. The system takes into account:

- frequency;

- startup sequence;

- configuration;

- priority between units.

The protocol is therefore expected to support more than a simple presence announcement. It probably also supports identification, status, configuration, synchronization, control, and priority or role assignment.

## 4.4 Configuration Synchronization

DAB describes parameters that must be coordinated between units, including the pressure setpoint and other control parameters. A change made on one unit can be synchronized with the other units.

unit A

│

│ parameter change

▼

CAN bus

│

├────────► unit B

├────────► unit C

└────────► ...

This is an important subject for later investigation.

## 4.5 Product Identification

Product types and product information form part of the DConnect communication and configuration process. It is reasonable to assume that a connected unit makes its identity or product type available at some point.

The following remains unknown:

- which CAN ID is used;

- whether the product type fits in one frame;

- whether request/response communication is used;

- whether the information is available only through DConnect;

- whether the same information is used during multi-inverter discovery.

## 4.6 Firmware Information

DAB documentation treats software and firmware versions as relevant information, including in multi-inverter configurations. Firmware version may therefore be part of unit identification or compatibility management.

Unknown elements include the CAN ID, payload format, encoding, byte order, checksum or CRC, and the conditions under which firmware information is exchanged.

# 5. Current Knowledge and Unknowns

| Item | Status |
| --- | --- |
| CAN physical layer | Probably confirmed |
| A/B communication conductors | Confirmed by DAB connection documentation |
| GND/shield | Confirmed |
| 120 Ω termination | Confirmed for the DConnect cable |
| Multi-inverter communication | Confirmed |
| Automatic detection | Confirmed as a function |
| Addressing | Confirmed as a function |
| Multi-inverter configuration | Confirmed |
| Leader/priority functionality | Documented |
| Product identification | Indicated by DAB documentation |
| Firmware information | Documented as relevant |
| CAN bit rate | Unknown |
| CAN IDs | Unknown |
| Payload structure | Unknown |
| Discovery frames | Unknown |
| Address-assignment frames | Unknown |
| Request/response format | Unknown |
| Counters | Unknown |
| Checksums/CRC | Unknown |
| Startup state machine | Unknown |
| CANopen | Not demonstrated |

Important: CANopen should not be assumed as the starting point. No reliable evidence has been found that the protocol is CANopen. A proprietary application layer implemented on top of CAN remains a serious possibility.

# 6. Available Test Equipment

| Device under test | DAB Active Driver Plus M/M 1.5 |
| --- | --- |
| CAN interface | PEAK PCAN-USB |
| Operating system | Windows 10 |
| Initial software | PCAN-View |
| Preferred file format | Original PCAN-View .TRC |

Initial captures should not be converted to Excel or manually modified. The original trace is the authoritative source.

# 7. Initial Investigation Strategy

## 7.1 General Principle

The first objective is not to guess CAN IDs. A repeatable startup baseline must be established before semantic interpretation begins.

## 7.2 Capture 1 — Normal Power Cycle

T = -5 s    Device powered off

Start CAN recording

POWER ON

T = 0       Complete startup

Continue logging for at least 30–60 s

## 7.3 Captures 2 Through 10

Repeat the same procedure multiple times, preserving the original trace files. Compare:

- CAN IDs that always occur;

- CAN IDs that occur only during startup;

- timing and frequency;

- payload structure;

- repetition and retry patterns;

- time-outs;

- relationships between frames.

## 7.4 Initial Analysis Format

ID       DLC    frequency    first appearance

------------------------------------------------

0x???     8       ??? Hz      startup

0x???     2       ??? Hz      startup

0x???     8       ??? Hz      runtime

...

For each ID, analyze byte variability, bit changes, periodicity, possible counters, possible checksums, and relationships with other IDs.

# 8. Priority Experiments

## 8.1 Automatic Versus Manual Addressing

Because only one Active Driver is available, the highest-priority controlled comparison is automatic addressing versus manual addressing:

Capture A: AD = AUTO

Capture B: AD = 1

Capture C: AD = 2

Capture D: AD = 3

This should only be performed if the configuration change is safe according to the DAB instructions. A CAN ID, byte field, or sequence that changes consistently with AD would be a strong candidate for the addressing mechanism.

## 8.2 DConnect-Assisted Capture

A DConnect Box could potentially provide an external communication partner and generate request/response traffic that does not occur in standalone operation.

DConnect Box

│

│ CAN

│

Active Driver Plus

│

PCAN-USB

(passive)

This could reveal device discovery, product identification, parameter reading, parameter writing, and firmware/version information.

# 9. Analysis Methodology and Role of AI

AI should be used only after the raw captures are available. The desired pipeline is:

PCAN-View

│

▼

.TRC

│

▼

Python parser

│

├── CAN ID analysis

├── timing analysis

├── payload analysis

├── clustering

├── sequence detection

└── correlation

│

▼

AI analysis

│

▼

hypotheses

│

▼

human verification

AI should formulate evidence-based hypotheses rather than assign unsupported meanings to identifiers.

Example evidence-based hypothesis: “ID 0x123 appears only during startup, is transmitted five times at intervals of approximately 50 ms, and is followed by ID 0x456. This resembles a request/retry sequence.”

# 10. Open Technical Questions

- What CAN bit rate does the Active Driver Plus use?

- Which pin is A?

- Which pin is B?

- Which pin is GND/shield?

- What termination is required for a single unit?

- Which CAN IDs appear during startup?

- Is there traffic when only one unit is present?

- Which frames are repeated?

- Is there a discovery request without a response?

- How is AD represented in the communication?

- How is N determined?

- Is there a broadcast ID?

- Is the node ID contained in the CAN ID or in the payload?

- Is there a counter?

- Is there a checksum or CRC?

- Does the protocol use request/response communication, broadcasts, or both?

- Which messages appear only after the unit becomes operational?

- What information can be requested through DConnect?

- Is the multi-inverter protocol the same as the DConnect protocol?

- Is the protocol proprietary or based on a known CAN application layer?

# 11. Current Working Hypothesis

DAB uses a CAN physical layer with a proprietary application protocol for Active Driver Plus systems.

The protocol appears to require support for at least:

Discovery

↓

Addressing

↓

Identification

↓

Multi-inverter configuration

↓

Leader/priority

↓

Runtime synchronization/control

This is a working hypothesis, not a proven protocol description. The actual CAN IDs, frames, and payloads must be established through measurement.

# 12. Starting Point for the Next Session

Available hardware:

- DAB Active Driver Plus M/M 1.5

- PEAK PCAN-USB

- Windows 10 computer

Preferred initial software: PCAN-View

Immediate objective: Create a completely passive startup capture in the original .TRC format.

Subsequent analysis should begin from the unmodified raw CAN data.

# 13. References

- DAB Active Driver Plus official product page:
https://www.dabpumps.com/en/products/command-and-control-system/inverter/Active_Driver_Plus

- Official DAB DConnect Box documentation:
https://www.dabpumps.com/sites/default/files/document/60183883-DCONNECT-Instructions_2.pdf

- Official DAB Active Driver Plus training:
https://www.dabpumps.com/en/d.training/course-catalog/activedriver-plus