Reconstruction of the Original Basis for Selecting Token C82D

DAB Active Driver Plus CAN Reverse-Engineering Project

Historical reconstruction based on the December 2025 captures

1. Executive Summary

The origin of the choice of C82D can be reconstructed with reasonable confidence, although no available note explicitly states: “C82D was selected for reason X.”

The available captures indicate that C82D was first observed as a naturally transmitted token from the controller. It appeared as a valid long-and-short RX frame pair in captures made with both AD=1 and AD=2. It was subsequently reused deliberately as a fixed token in active transmissions in Test_009.CSV.

The most likely rationale was therefore:

Use a token that the real controller had demonstrably transmitted, rather than an entirely arbitrary value, and hold that token constant while changing the CAN family or class field.

There is no evidence that C82D is a special command, discovery opcode, or “magic value.” It is best understood as a known-valid reference token.

2. First Natural Observation with AD=1

In Test_001_AD-1.CSV, recorded on 12 December 2025, the following natural RX frame pair appears:

17987)  44960.0  Rx  0001C82D  7  01 00 00 0F 03 2D C8

17988)  44960.2  Rx  0011C82D  5  01 00 00 2D C8

This demonstrated that the controller itself used the following combination:

Token in CAN identifier:  C82D

Token in payload:         2D C8

The observation therefore established that C82D, with payload bytes 2D C8, was syntactically valid within the observed protocol.

3. Repeated Natural Observation with AD=2

In Test_003_AD-2.CSV, recorded on 14 December 2025, the same natural RX pair appears again:

12030)  34733.8  Rx  0001C82D  7  01 00 00 0F 03 2D C8

12031)  34734.0  Rx  0011C82D  5  01 00 00 2D C8

This was significant because the same token was observed in captures associated with two different AD settings:

AD=1 in Test_001_AD-1.CSV;

AD=2 in Test_003_AD-2.CSV.

At the time, this would have made C82D a useful candidate for a neutral test token: it appeared to be a valid dynamic token rather than a value tied exclusively to one AD setting.

4. Deliberate Reuse in Test_009

In Test_009.CSV, recorded later on 14 December 2025, C82D appears in active TX frames rather than only in naturally received traffic.

4.1 Initial TX family

0011C82D  5  01 00 00 2D C8

4.2 Alternative family

0021C82D  5  01 00 00 2D C8

4.3 Extended test using family 0012

0012C82D  5  01 00 00 2D C8

The capture contains 76 deliberate transmissions using token C82D:

CAN identifierNumber of transmissionsGeneral pattern

0011C82D9Initial test phase

0021C82D6Alternative family/class test

0012C82D61Long-running test, approximately one transmission every five seconds

This establishes that C82D had been deliberately selected as a fixed reference token by the time Test_009.CSV was recorded.

5. Most Likely Original Selection Rationale

The probable decision process was:

Identify a token naturally produced by the controller.

Verify the correct relationship between the CAN identifier and payload bytes.

Prefer a token seen under more than one AD configuration.

Keep the token and payload constant while changing the upper family/class field.

This approach avoids introducing two variables at once. With token C82D fixed, an experiment can change only the family:

Variable:   family/class field

Constant:   token C82D

Constant:   payload 01 00 00 2D C8

A resulting device reaction can then be attributed more confidently to the changed family rather than to a simultaneous token change.

6. Advantages of C82D as a Reference Token

6.1 Demonstrably protocol-valid

The controller had naturally transmitted both:

0001C82D  01 00 00 0F 03 2D C8

0011C82D  01 00 00 2D C8

This meant that neither the token nor its payload representation had to be guessed.

6.2 Known byte order

The relationship was unambiguous:

Token value:       C82D

Payload encoding:  2D C8

Byte order:         little-endian

6.3 Seen under multiple AD settings

The token appeared naturally in captures labeled AD=1 and AD=2. This suggested that it was not a permanent representation of one specific configured address.

6.4 Suitable for controlled experiments

Keeping the token fixed allowed the family field to be varied independently. This later became especially valuable in the September 2026 family-probe tests.

7. What C82D Probably Does Not Represent

The available evidence does not support interpreting C82D as:

a discovery opcode;

a permanent device address;

a product identifier;

a fixed command;

a privileged or authenticated token;

a protocol “magic value.”

Normal traffic contains a rapidly changing 16-bit token stream. The natural appearances of C82D are consistent with it being one value from that dynamic token space.

The current interpretation is therefore:

C82D = known-valid reference token

rather than:

C82D = special discovery command or special address

8. Confirmation from the September 2026 Tests

The later family-probe tests retained token C82D while changing the CAN family. The results showed that the device reaction depended on the selected family and operating state:

some families generated a display change and a 103C response burst;

other families generated no such response;

the same token was present in every probe.

This confirms that C82D alone was not responsible for the application response. It functioned effectively as a controlled constant in the experiments.

9. Historical Reconstruction

DateCaptureObservation

12 December 2025Test_001_AD-1.CSVNatural RX pair using token C82D under AD=1

14 December 2025Test_003_AD-2.CSVNatural RX pair using the same token under AD=2

14 December 2025Test_009.CSVC82D deliberately reused in 76 active transmissions across families 0011, 0021, and 0012

28 September 2026dab_family_probe_*.csvC82D retained as the fixed token while testing multiple family values

10. Conclusion

The available evidence supports the following reconstruction:

C82D was probably selected in December 2025 because it had already been naturally transmitted by the controller, had a proven identifier-to-payload relationship, and had appeared under both AD=1 and AD=2. It was therefore a known-valid, comparatively low-risk reference value for active probes. Its purpose was to hold the token constant while other protocol fields were varied.

Test_009.CSV confirms that the token was subsequently used deliberately as a fixed test value. Nothing currently indicates that C82D has a unique protocol meaning beyond being a valid member of the changing token space.

11. Sources

Test_001_AD-1.CSV

Test_003_AD-2.CSV

Test_009.CSV

Safe discovery probes.docx

DAB Active Driver Plus CAN Reverse Engineering Test Plan.docx